Development of a UAV Multispectral Data Processing Pipeline for Navigation-Oriented Spatial Analysis

Abstract:This includes the development and validation of a software pipeline that transforms raw unmanned aerial vehicle (UAV) multispectral imagery into navigation-relevant spatial representations. The workflow integrates photogrammetric processing using WebODM and geospatial analysis using QGIS. Key outputs include georeferenced orthomosaics, vegetation indices (NDVI and NDRE), and a rule-based costmap for terrain traversability assessment. The results demonstrate a successful transition from raw UAV data to spatial features suitable for autonomous navigation research.

- Introduction

## 1.1 Background

With the rapid development of unmanned aerial vehicles (UAVs) and multispectral sensing technologies, high-resolution remote sensing data has become increasingly valuable for environmental monitoring, precision agriculture, and autonomous robotic navigation. However, raw UAV imagery cannot be directly used for spatial analysis or navigation due to geometric distortions and lack of georeferencing.

To address this, a structured processing pipeline is required to convert raw UAV images into geospatially accurate and analytically meaningful data products. This report focuses on establishing such a pipeline using open-source tools.

1.2 Objectives

The main objectives of this milestone are:

- To deploy and understand the WebODM photogrammetry workflow.

- To generate georeferenced orthomosaic data from UAV imagery.

- To perform multispectral raster analysis in QGIS.

- To compute vegetation indices (NDVI and NDRE) using real spectral bands.

- To construct a baseline costmap representing terrain traversability.

- To establish a clear software pipeline supporting future machine learning-based navigation mapping.

## 2.System Architecture and Workflow

The developed processing pipeline is structured as follows:

Raw UAV Images

→ WebODM (Photogrammetry Processing)

→ Orthomosaic & GeoTIFF Outputs

→ QGIS (Raster Analysis)

→ NDVI & NDRE Calculation

→ Rule-Based Costmap Generation

→ (Future) Machine Learning for Traversability Prediction

2.1 Role of Each Component

| Component | Function |
| --- | --- |
| WebODM / OpenDroneMap | Converts overlapping UAV images into georeferenced orthomosaics and terrain products |
| QGIS | Performs raster visualization, band management, and spatial analysis |
| Vegetation Indices (NDVI, NDRE) | Quantify vegetation health and density using multispectral data |
| Costmap | Provides a spatial representation of terrain traversability for navigation |
| Future ML/DL Models | Predict traversability based on spectral and spatial features |

- 3. WebODM Deployment and Photogrammetric Processing

3.1 Environment Setup

WebODM was successfully deployed locally using Docker and WSL2 on a Windows system. The setup process included:

- Installation and configuration of Docker Desktop

- Enabling WSL2 backend

- Downloading and launching WebODM using the webodm.sh script

- Accessing the WebODM interface via http://localhost:8000

All required containers (webapp, worker, db, broker, and node-odm) were successfully initialized and run correctly.

3.2 Workflow Understanding

Through practical experimentation, the internal photogrammetric workflow of WebODM was clearly understood:

- Image alignment and feature matching

- Sparse point cloud generation

- Dense reconstruction

- Orthorectification and orthomosaic generation

The most critical output for this project is the orthomosaic in GeoTIFF format, which provides spatially accurate imagery for further analysis in QGIS.

3.3 Challenges and Observations

During processing, tasks frequently failed due to insufficient memory, even with moderate image counts. This led to several key insights:

- Photogrammetric reconstruction is highly resource-intensive.

- Local laptop environments have limitations when handling large datasets.

- Proper memory allocation via WSL2 configuration and smaller test datasets are necessary for stable processing.

Despite these constraints, the successful deployment and workflow validation confirm that WebODM is a reliable preprocessing tool within the pipeline.

- 4. Multispectral Data Characteristics

The multispectral dataset used in this project includes the following bands:

- Green (G)

- Red (R) (extracted from RGB imagery)

- Red Edge (RE)

- Near Infrared (NIR)

- RGB Orthomosaic

These spectral bands are essential for computing vegetation indices such as NDVI and NDRE, which are widely used to assess vegetation health, density, and stress levels—key indicators for terrain traversability in agricultural environments.

- 5. QGIS-Based Multispectral Raster Analysis

5.1 Data Import and Band Interpretation

Georeferenced multispectral raster datasets were successfully imported into QGIS, including NIR, RE, RGB, and NDVI reference layers. Using QGIS raster properties and visualization tools, the following were achieved:

- Identification of raster band structures

- Differentiation between single-band and multi-band rasters

- Verification of spatial alignment across all spectral layers

This step ensured the correctness and consistency of input data for subsequent analysis.

5.2 NDVI Calculation

The Normalized Difference Vegetation Index (NDVI) was calculated using the formula:

NDVI=NIR−RNIR+R

Since the red band was embedded within the RGB orthomosaic, the red channel was extracted and used in combination with the NIR band in the QGIS Raster Calculator. The resulting NDVI map effectively highlights vegetation vigor and density across the study area.

5.3 NDRE Calculation

The Normalized Difference Red Edge Index (NDRE) was computed using:

NDRE=NIR−RENIR+RE

NDRE is particularly sensitive to chlorophyll content and is more effective than NDVI in detecting subtle variations in dense vegetation. The generated NDRE map provides enhanced insight into crop health and canopy conditions.

- 6. Key Achievements

6.1 Technical Achievements

- Successfully deployed and configured WebODM in a local environment.

- Understood and validated the UAV photogrammetry workflow.

- Imported and analyzed real multispectral orthomosaic data in QGIS.

- Computed NDVI and NDRE using authentic spectral bands.

- Generated a rule-based baseline costmap for navigation analysis.

6.2 Conceptual Achievements

- Established a coherent and logical pipeline from raw UAV imagery to navigation-relevant spatial representations.

- Clearly defined the roles of WebODM and QGIS within the processing chain.

- Bridged remote sensing data processing with robotic navigation requirements

- 7. Challenges and Lessons Learned

7.1 Challenges

- Memory limitations during WebODM processing on a local machine.

- Complexity in configuring Docker and WSL2 resource allocation.

- Managing large multispectral datasets efficiently.

- Data collection limited

7.2 Lessons Learned

- Photogrammetric processing requires careful planning of computational resources.

- QGIS provides a powerful platform for multispectral raster analysis and spatial modeling.

- Incremental testing with smaller datasets is essential for stable system validation.

- A modular software pipeline enhances flexibility and scalability for future development.

- 8. Future Work

8.1 Workflow Automation

- Develop Python-based scripts using GDAL, Rasterio, or PyQGIS.

- Automate orthomosaic processing, vegetation index computation, and costmap generation.

8.2 Machine Learning-Based Traversability Modeling

- Use NDVI, NDRE, and other spatial features as input variables.

- Train models to classify terrain into multiple traversability levels.

8.3 Integration with Robotic Navigation Systems

- Convert generated costmaps into formats compatible with robotic frameworks such as ROS.

- Support occupancy grids and layered costmaps for autonomous path planning.

- 9. Conclusion

This milestone successfully established and validated a comprehensive software pipeline that converts raw UAV multispectral imagery into navigation-relevant spatial representations. Through the integration of WebODM and QGIS, the project achieved:

- Photogrammetric generation of georeferenced orthomosaics

- Accurate computation of NDVI and NDRE from real multispectral data

- Development of a baseline costmap for terrain traversability analysis

The results provide a technical and conceptual foundation for future work in machine learning-based traversability prediction and autonomous navigation in agricultural environments.